SECTION 08310

# ACCESS DOORS AND FRAMES

** NOTE TO SPECIFIER ** Battic Door Energy Conservation Products; products.
 
 This section is based on the products of Battic Door Energy Conservation Products, which is located at:
P. O. Box 15  
Mansfield, MA 02048-0015
Tel: 508.320.9082
Fax: 508.339.4571
Email: info@batticdoor.com
Web: www.batticdoor.com 
[ Click Here ] for additional information.
 
 THE ATTIC ACCESS DOOR HATCH - FOR NEW AND RETRO-FIT CONSTRUCTION

Installs In Minutes Into a 22-1/2" x 30-1/2" Rough Opening Model 22 x 30  -  Perfect for Builders and Contractors for New and Existing Construction

Meets and Exceeds all Building Code Requirements

The E-Z Hatch Attic Access Door Panel Hatch System is an easy to install, easy to use, highly insulated door, panel, hatch system that installs in minutes and provides an air sealed, professional, high-quality, insulated access opening into the attic space.

Triple gaskets provide an air tight seal!

The core of the door is 10" thick and made of Expanded Polystyrene foam that provides a greater R-Value per inch than fiberglass.

The face of the door is gloss white, durable, and easy to clean.

The top gasket of closed cell insulation seals the top of the door. The gasket is fire rated per ASTM E84 for flame spread (FSI) 25 or less, smoke developed (SDI) 50 or less, (required by Code for exposed insulation) and meets code requirements.

The lightweight door weighs 10 pounds, and lifts up and out of the way for easy attic access.

No hinges - The door is designed to prevent interference with the roof joists.

Triple pre-installed weatherstrip gaskets ensure an air tight seal. Gasketing provided where the trim meets the ceiling, where the face of the door meets the trim, and where the top of the door meets the frame.

The lightweight door opens with little effort, allowing easy entry into your attic space. The finished face on the door fits tightly into the weatherstrip on the trim when the door closes.

Perfect for use with any type of attic insulation, including blown-in insulation. The 12 in. tall wood frame acts as an insulation dam, which holds the insulation back from the opening, and takes the mess out of getting into the attic space.

Sized to fit either between roof trusses 24" on-center, or between ceiling joists 16" o.c. with minimal carpentry required (cut one joist and add a header and footer).

Whether in new construction or retrofit, the unit can be installed in minutes with only a utility knife and a screwdriver.

Prefinished gloss white trim and door standard.

- GENERAL

  - RELATED SECTIONS

    - Section 06100 -  Rough Carpentry: Roof framing and opening support.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - [ Product Data ]: Manufacturer's data sheets, including:

      - Brochures.

      - Installation Instructions.

  - DELIVERY, STORAGE, AND HANDLING

    - Store attic access door hatch in manufacturer's unopened packaging until ready for installation.

    - Store attic access door hatch until installation inside under cover in dry area out of direct sunlight.

  - WARRANTY

    - Limited Warranty:  One year against defective material and workmanship, covering parts only, no labor or freight. Defective parts, if deemed so by the manufacturer, will be replaced at no charge, freight excluded, upon inspection at manufacturer's plant which warrants same.

- PRODUCTS

  - MANUFACTURERS

A.	Acceptable Manufacturer:

Battic Door Energy Conservation Products.

Box 15, Mansfield, MA 02048-0015 U.S.A.

Phone: (508) 320-9082; Fax: (508) 339-4571

e: info@batticdoor,com;

i:  www.batticdoor.com

  - FABRICATION

    - Manufacture each access panel assembly as an integral unit ready for installation.

2.3	OVERVIEW OF PRODUCT

A.	Model 22 x 30 Attic Access Door System is an R-42 insulated and triple-gasketed door that installs in minutes and provides an air sealed, insulated access opening to residential attic space. Door is provided with wood trim attached to the wood frame. The door face and trim are pre-finished satin white and the unit is shipped ready to install. Exceeds 2009, 2012 and 2015 IBC and IECC requirements.

An R-50 Version is also available.

2.4 	MATERIALS

A.	Wood frame and door. Unit installs into a 22-1/2” x 30-1/2” framed opening. Fits within the standard space between trusses spaced 24' o.c. or joists 16" o.c. to simplify installation. Pre-painted satin white door face and trim. R-42 insulated door core is 10" thick Extruded Polystyrene (EPS). Door face is mineral board facer and is painted satin white. Wood trim is attached to door frame and is painted satin white. Triple rubber gaskets provided for air tight seal. Solid wood frame and trim.

An R-50 Version is also available.

2.5	ACCESS DOOR AND FRAME FOR ATTIC ACCESS IN RESIDENTIAL APPLICATIONS

A.	Non-rated attic access doors and frames.

- Door:  White mineral board face, extruded polystyrene (EPS) R-42 core 10" thick, and fire rated rubber sealing gasket over top of EPS core. Door is pre-finished and painted satin white to match trim. R-50 Version also available.

- Frame:  Plywood frame is installed into a wood framed 22-1/2” x 30-1/2” rough opening. Frame is 12” tall and provides insulation dam. Wood trim is pre-secured to frame. Rubber gasket is installed on trim on each side of frame.

- Trim: Tapered wood trim 2" wide is pre-secured to frame and painted satin white to match door.

- Finish:  Pre-finished satin white painted wood door and trim.

.

- EXECUTION

  - EXAMINATION

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

    - Examine materials upon arrival at site.  Notify the carrier and manufacturer of any damage.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION